Claude Hardware Design Environment

Agent Setup Reference

This document provides step-by-step environment setup instructions for an autonomous agent to configure a complete text-prompt-to-hardware-schematic pipeline using Claude Code, KiCad, Diode Zener, and supporting MCP servers.

| Property | Value |
| --- | --- |
| Platform | Windows 11 + WSL2 (Kali/Ubuntu) — primary target |
| Primary agent | Claude Code (terminal alias) |
| EDA backend | KiCad 9.0+ |
| Schematic DSL | Diode Zener (built on Starlark) |
| Component sourcing | pcbparts-mcp, @jlcpcb/mcp |
| Optional EDA GUI | EasyEDA Pro (with MCP bridge) |
| Validation | Anthropic-Diode partnership workflow |

NOTE: Windows support for Diode pcb CLI is experimental. All shell commands in this document target WSL2 (Debian/Ubuntu base). Run everything inside WSL unless otherwise stated.

# 1. Prerequisites

Verify all of the following before proceeding. Missing items must be resolved first — do not skip ahead.

## 1.1 WSL2 Environment

# Confirm WSL2 (not WSL1)

wsl --list --verbose

# VERSION column must show 2 for your distro

# If not WSL2, upgrade:

wsl --set-version <DistroName> 2

## 1.2 Required System Packages

sudo apt-get update && sudo apt-get install -y \

git curl wget build-essential python3 python3-pip \

nodejs npm unzip ca-certificates

NOTE: Node.js from apt is often outdated. See Section 1.3 to install v20.x via NodeSource.

## 1.3 Node.js v20+

curl -fsSL https://deb.nodesource.com/setup_20.x ¦ sudo -E bash -

sudo apt-get install -y nodejs

node --version # must be v20.x or higher

npm --version

## 1.4 Python 3.8+

python3 --version # must be 3.8 or higher

pip3 --version

## 1.5 Rust Toolchain (required for Diode pcb CLI)

curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs ¦ sh -s -- -y

source $HOME/.cargo/env

rustc --version

## 1.6 Claude Code

Claude Code must already be installed on Windows and accessible via a shell alias in WSL. Verify:

claude --version # should resolve via your alias to the .exe

NOTE: If not set up, the alias should point to: /mnt/c/Users/<username>/.local/bin/claude.exe. Add to ~/.bashrc or ~/.zshrc: alias claude='/mnt/c/Users/<username>/.local/bin/claude.exe'

# 2. KiCad 9.0 Installation

KiCad 9.0 is required. Earlier versions lack the Python API features used by the MCP servers. Install on Windows (not WSL) and verify the Python API is accessible from WSL.

## 2.1 Install KiCad on Windows

- Download KiCad 9.0 installer from kicad.org/download/windows

- Run the installer with default options — ensure Python scripting is included

- After install, locate the Python bundled with KiCad:

# On Windows (run in cmd.exe or PowerShell, not WSL):

C:\Program Files\KiCad\9.0\bin\python.exe --version

## 2.2 Verify Python API from WSL

# Set PYTHONPATH to KiCad's Python packages via WSL path

export PYTHONPATH='/mnt/c/Program Files/KiCad/9.0/lib/python3/dist-packages'

python3 -c "import pcbnew; print(pcbnew.GetBuildVersion())"

# Expected: version string like 9.0.x

NOTE: If the import fails, check that the KiCad Python path is correct. The exact path may vary by KiCad install version. Add the export to ~/.bashrc for persistence.

## 2.3 Install kicad-cli for Headless DRC

kicad-cli is bundled with KiCad 9.0 on Windows. Accessible from WSL:

alias kicad-cli='/mnt/c/Program Files/KiCad/9.0/bin/kicad-cli.exe'

kicad-cli --version

# 3. Diode Zener + pcb CLI (Primary Schematic Engine)

This is the Anthropic-validated, Claude-native path. Diode Computers partnered with Anthropic to develop this pipeline. Claude Sonnet 4.5 was benchmarked against this toolchain — Diode engineers preferred its output 8/10 times in blind evaluations.

## 3.1 What It Is

- Zener: open-source DSL built on Starlark (strict Python subset) for describing PCB schematics as code

- pcb CLI: compiles Zener .zen files into KiCad projects ready for layout and fabrication

- Workflow: text prompt → Claude writes .zen file → pcb build validates → pcb layout opens KiCad

- 250+ published reference designs at zener.diode.computer covering sensors, MCUs, power stages, analog chains

## 3.2 Install pcb CLI

# Clone the repository

git clone https://github.com/diodeinc/pcb.git ~/pcb

cd ~/pcb

# Run the install script (builds from source using Rust)

./install.sh

# Verify

pcb --version

NOTE: The install script requires Rust (see Section 1.5). Build time is approximately 2-5 minutes on first run.

## 3.3 Zener Language Basics for Agent Context

Provide this context block to Claude Code at the start of any hardware design session:

# Zener primitives Claude needs to know:

# Component(symbol=..., ref=..., value=..., footprint=...) -- place a part

# Net('NAME') -- declare a net

# component['PIN'] += net -- connect pin to net

# @module def my_block(vcc, gnd): ... -- reusable sub-circuit

# Example: 3.3V LDO from 5V

load('//components/regulators/ams1117-3.3.zen', 'AMS1117_3V3')

@module

def power_3v3(vbus, v3v3, gnd):

reg = AMS1117_3V3()

reg['IN'] += vbus

reg['OUT'] += v3v3

reg['GND'] += gnd

cap_in = Component(symbol='Device:C', value='10uF', ref='C')

cap_out = Component(symbol='Device:C', value='22uF', ref='C')

cap_in[1] += vbus; cap_in[2] += gnd

cap_out[1] += v3v3; cap_out[2] += gnd

## 3.4 pcb CLI Command Reference

| Command | Description |
| --- | --- |
| pcb build [PATH] | Build and validate .zen files — reports ERC and type errors |
| pcb layout [PATH] | Generate KiCad project and open in KiCad for layout |
| pcb open [PATH] | Open existing KiCad layout without regenerating |
| pcb fmt [PATH] | Format .zen files using ruff fmt |

## 3.5 Agent Workflow: Text Prompt to Schematic

Instruct Claude Code to follow this sequence for any hardware design request:

- Create a project directory: mkdir ~/designs/<project_name> && cd ~/designs/<project_name>

- Query pcbparts-mcp to find real in-stock components matching the design requirements

- Write a main.zen file describing the circuit using Zener syntax, referencing components by LCSC/JLCPCB part numbers

- Run: pcb build main.zen — iterate on errors until clean

- Run: pcb layout main.zen — generates KiCad .kicad_pro, .kicad_sch files

- Pass KiCad project to mixelpixx/KiCAD-MCP-Server for DRC, BOM export, and Gerber generation

# 4. MCP Server Setup

Configure MCP servers in Claude Desktop or Claude Code. Config file locations:

# Claude Desktop (Windows):

%APPDATA%\Claude\claude_desktop_config.json

# Claude Code (WSL) — project-level:

.claude/claude_desktop_config.json

# Claude Code (WSL) — global:

~/.config/Claude/claude_desktop_config.json

## 4.1 pcbparts-mcp (Component Sourcing — No Auth Required)

Remote HTTP MCP server. No installation. Covers JLCPCB, Mouser, DigiKey. 1.5M+ parts, parametric filtering, KiCad footprint downloads, 285 OSHW reference boards, 41 PCB design rule files.

Add to MCP config:

{

"mcpServers": {

"pcbparts": {

"command": "npx",

"args": ["-y", "mcp-remote", "https://pcbparts.dev/mcp"]

}

}

}

Example queries Claude can issue to this server:

- "Find logic-level MOSFETs with Vgs(th) < 2V and Id >= 5A in stock at JLCPCB"

- "Search for STM32F411 microcontrollers — show pricing and stock"

- "Get KiCad footprint for ESP32-S3-WROOM-1"

- "What sensor ICs measure CO2 with I2C interface compatible with ESP32?"

- "Show me how the MCP73831 charger IC is typically used in reference boards"

## 4.2 @jlcpcb/mcp (JLCPCB Library + KiCad Symbols)

Anthropic's own EDA MCP from the anthropics/ai-eda repo. Fetches components from JLCPCB/EasyEDA library and installs them as KiCad-compatible symbols and footprints.

# Install

git clone https://github.com/anthropics/ai-eda.git ~/ai-eda

cd ~/ai-eda/packages/jlc-mcp

bun install && bun run build

# Or install bun first if not present:

curl -fsSL https://bun.sh/install ¦ bash

Add to MCP config:

{

"mcpServers": {

"jlc": {

"command": "node",

"args": ["/absolute/path/to/ai-eda/packages/jlc-mcp/dist/index.js"]

}

}

}

NOTE: Libraries install to ~/Documents/KiCad/9.0/3rdparty/jlc_mcp/ using portable ${KICAD9_3RD_PARTY} paths. Up to 10 components can be installed in parallel.

## 4.3 mixelpixx/KiCAD-MCP-Server (Full KiCad Control)

TypeScript MCP server that controls KiCad 9.0 via the pcbnew Python API. Used for DRC, ERC, BOM export, Gerber generation, custom symbol creation, and Freerouting integration.

# Install

git clone https://github.com/mixelpixx/KiCAD-MCP-Server.git ~/KiCAD-MCP-Server

cd ~/KiCAD-MCP-Server

npm install

pip3 install -r requirements.txt

npm run build

Add to MCP config (WSL):

{

"mcpServers": {

"kicad": {

"command": "node",

"args": ["/home/<username>/KiCAD-MCP-Server/dist/index.js"],

"env": {

"NODE_ENV": "production",

"PYTHONPATH": "/mnt/c/Program Files/KiCad/9.0/lib/python3/dist-packages",

"LOG_LEVEL": "info"

}

}

}

}

Capabilities covered by this server:

- Project setup, schematic editing, component placement, routing

- DRC (Design Rule Check) and ERC (Electrical Rule Check)

- Custom symbol and footprint generation for parts not in standard libraries

- JLCPCB parts catalog with pricing and stock data

- Freerouting integration for automatic PCB routing via Java/Docker

- Export: Gerber files, PDF documentation, SVG, 3D STEP/VRML models, BOM

## 4.4 Seeed-Studio KiCad MCP (Embedded Dev Focus)

39 tools across 7 categories. Strongest for MCU-heavy designs — generates device trees (.dts) for STM32, pin conflict detection, and C test code alongside the schematic.

git clone https://github.com/Seeed-Studio/kicad-mcp-server.git ~/seeed-kicad-mcp

cd ~/seeed-kicad-mcp

pip3 install -r requirements.txt

Add to MCP config:

{

"mcpServers": {

"kicad-seeed": {

"type": "stdio",

"command": "python3",

"args": ["-m", "kicad_mcp_server"],

"cwd": "/home/<username>/seeed-kicad-mcp",

"env": {

"PYTHONPATH": "/home/<username>/seeed-kicad-mcp/src"

}

}

}

}

## 4.5 EasyEDA Pro MCP (Optional — Full GUI Automation)

Use this if EasyEDA Pro (free desktop app) is installed on Windows. Provides 72 MCP tools and a 267-method WebSocket bridge. Enables fully autonomous design from component search through manufacturing export without leaving natural language.

Prerequisites: EasyEDA Pro desktop app installed on Windows. Download from easyeda.com/page/download.

git clone https://github.com/teileelektronik/easyeda-mcp.git ~/easyeda-mcp

cd ~/easyeda-mcp

npm install && npm run build

# Build the bridge extension

# Then in EasyEDA Pro: Top Menu > Settings > Extensions > Extension Manager > Import

# Select: ~/easyeda-mcp/src/bridge/dist/

Add to MCP config:

{

"mcpServers": {

"easyeda": {

"command": "node",

"args": ["/home/<username>/easyeda-mcp/dist/server/index.js"],

"env": {

"ANTHROPIC_API_KEY": "sk-ant-..."

}

}

}

}

NOTE: No LCSC/JLCPCB API key needed — component search uses public endpoints. If both ANTHROPIC_API_KEY and GEMINI_API_KEY are set, Claude takes priority. The MCP server starts a WebSocket listener on localhost:18601 that the EasyEDA Pro bridge connects to.

Example full-board prompt for EasyEDA MCP:

Design a USB-C powered ESP32-S3 board with:

- USB-C with PD sink (5V/9V)

- 3.3V LDO from VBUS

- SPI TFT display connector (4-wire SPI)

- I2S DAC (PCM5102A) for audio output

- 3 tactile buttons with debouncing

- WS2812B RGB LED

Run full DRC on both schematic and PCB.

Export manufacturing files (Gerbers, BOM, pick-and-place).

# 5. Claude Code Skills — kicad-happy

aklofas/kicad-happy provides 12 Claude Code skills for KiCad hardware design, validated against 5,800+ open-source KiCad projects. These install as agent skills directly, not as MCP servers.

## 5.1 Install

git clone https://github.com/aklofas/kicad-happy.git ~/kicad-happy

cd ~/kicad-happy

# Install all 12 skills into Claude Code's skills directory

mkdir -p ~/.claude/skills

for skill in kicad spice emc datasheets bom digikey mouser lcsc element14 jlcpcb pcbway kidoc; do

ln -sf "$(pwd)/skills/$skill" ~/.claude/skills/$skill

done

## 5.2 Skill Reference

| Skill | Function | Notes |
| --- | --- | --- |
| kicad | Schematic/PCB analysis, layout review | Core skill — always load |
| spice | SPICE simulation and analysis | Requires ngspice or LTspice |
| emc | EMC pre-compliance checks | References IPC-2141, IPC-7711 |
| datasheets | Download datasheets from LCSC, DigiKey, Mouser | Free tier: LCSC only |
| bom | BOM generation and management | Outputs CSV/JSON |
| digikey | DigiKey component search | Requires DigiKey API key |
| mouser | Mouser component search | Requires Mouser API key |
| lcsc | LCSC component search | Free, no key required |
| element14 | Element14/Farnell search | No key required |
| jlcpcb | JLCPCB parts and pricing | No key required |
| pcbway | PCBWay DFM analysis | No key required |
| kidoc | KiCad documentation lookup | Local reference |

NOTE: Two-tier PR review: deterministic analysis runs free with no API key. AI-powered review requires ANTHROPIC_API_KEY set in environment.

# 6. circuit-synth (Python-Based Alternative)

circuit-synth provides a Python-native circuit definition approach with KiCad integration. Useful when Claude Code's Python generation capabilities are preferred over Zener DSL. Fully bi-directional — can import existing KiCad schematics into Python for modification.

## 6.1 Install

pip3 install circuit-synth

# Create a new project with working example

cs-new-project my_first_board

cd my_first_board

# Generate KiCad files from the example

python3 main.py

# Open in KiCad (generates project in ESP32_C6_Dev_Board/)

pcb open ESP32_C6_Dev_Board/ESP32_C6_Dev_Board.kicad_pro

## 6.2 Circuit Definition Pattern

from circuit_synth import *

@circuit(name='Power_Supply')

def power_supply(vbus_in, vcc_3v3_out, gnd):

regulator = Component(

symbol='Regulator_Linear:AMS1117-3.3',

ref='U',

footprint='Package_TO_SOT_SMD:SOT-223-3_TabPin2'

)

cap_in = Component(symbol='Device:C', ref='C', value='10uF',

footprint='Capacitor_SMD:C_0805_2012Metric')

cap_out = Component(symbol='Device:C', ref='C', value='22uF',

footprint='Capacitor_SMD:C_0805_2012Metric')

regulator['VI'] += vbus_in

regulator['VO'] += vcc_3v3_out

regulator['GND'] += gnd

cap_in[1] += vbus_in; cap_in[2] += gnd

cap_out[1] += vcc_3v3_out; cap_out[2] += gnd

@circuit(name='Main_Circuit')

def main_circuit():

vbus = Net('VBUS')

v3v3 = Net('VCC_3V3')

gnd = Net('GND')

power_supply(vbus, v3v3, gnd)

if __name__ == '__main__':

circuit = main_circuit()

circuit.generate_kicad_project('my_board')

NOTE: Use circuit-synth when the prompt involves repeated subcircuit blocks or systematic modifications to existing KiCad files. Use Zener/pcb when operating from reference designs or chip datasheets.

# 7. EDA Tools MCP Server (FPGA / ASIC Path)

For Verilog synthesis, simulation, ASIC design flows, and waveform analysis. Separate from the PCB design stack — use for FPGA target hardware or digital logic verification.

## 7.1 Required Backend Tools

# Yosys (synthesis)

sudo apt-get install -y yosys

# iverilog (simulation)

sudo apt-get install -y iverilog

# For iCE40 FPGA targets (optional):

sudo apt-get install -y nextpnr-ice40 icestorm

## 7.2 MCP Config Entry

{

"mcpServers": {

"eda-tools": {

"command": "npx",

"args": ["-y", "eda-mcp"]

}

}

}

Example Claude prompt using this server:

Synthesize this Verilog counter module for an iCE40 FPGA:

module counter(

input clk, input rst, output [7:0] count

);

reg [7:0] count_reg;

assign count = count_reg;

always @(posedge clk or posedge rst) begin

if (rst) count_reg <= 8'b0;

else count_reg <= count_reg + 1;

end

endmodule

# 8. Complete MCP Configuration File

Paste this into your Claude Desktop or Claude Code config file. Replace all <username> and path placeholders with actual values before use.

{

"mcpServers": {

"pcbparts": {

"command": "npx",

"args": ["-y", "mcp-remote", "https://pcbparts.dev/mcp"]

},

"jlc": {

"command": "node",

"args": ["/home/<username>/ai-eda/packages/jlc-mcp/dist/index.js"]

},

"kicad": {

"command": "node",

"args": ["/home/<username>/KiCAD-MCP-Server/dist/index.js"],

"env": {

"NODE_ENV": "production",

"PYTHONPATH": "/mnt/c/Program Files/KiCad/9.0/lib/python3/dist-packages",

"LOG_LEVEL": "info"

}

},

"kicad-seeed": {

"type": "stdio",

"command": "python3",

"args": ["-m", "kicad_mcp_server"],

"cwd": "/home/<username>/seeed-kicad-mcp",

"env": {

"PYTHONPATH": "/home/<username>/seeed-kicad-mcp/src"

}

},

"eda-tools": {

"command": "npx",

"args": ["-y", "eda-mcp"]

}

}

}

NOTE: easyeda-mcp is excluded from this combined config because it requires EasyEDA Pro to be running first. Add it separately when using the EasyEDA workflow.

# 9. Environment Verification

Run all of the following checks after setup. All must pass before beginning hardware design work.

## 9.1 Full Verification Script

#!/bin/bash

set -e

echo '=== Claude Hardware Design Environment Check ==='

echo '[1] Node.js...'

node --version ¦ grep -E 'v(1[89]¦[2-9][0-9])\.' ¦¦ (echo 'FAIL: Node < 18' && exit 1)

echo '[2] Python...'

python3 -c 'import sys; assert sys.version_info >= (3,8)' ¦¦ (echo 'FAIL: Python < 3.8' && exit 1)

python3 --version

echo '[3] Rust...'

rustc --version ¦¦ (echo 'FAIL: Rust not installed' && exit 1)

echo '[4] KiCad Python API...'

python3 -c 'import pcbnew; print(pcbnew.GetBuildVersion())' ¦¦ (echo 'FAIL: KiCad Python API not found' && exit 1)

echo '[5] pcb CLI (Diode Zener)...'

pcb --version ¦¦ (echo 'FAIL: pcb CLI not in PATH' && exit 1)

echo '[6] KiCAD-MCP-Server build...'

test -f ~/KiCAD-MCP-Server/dist/index.js ¦¦ (echo 'FAIL: KiCAD-MCP-Server not built' && exit 1)

echo '[7] circuit-synth...'

python3 -c 'import circuit_synth' ¦¦ (echo 'WARN: circuit-synth not installed' )

echo '[8] Claude Code...'

claude --version ¦¦ (echo 'FAIL: claude alias not configured' && exit 1)

echo ''

echo '=== All checks passed ==='

Save as ~/verify_hardware_env.sh, then:

chmod +x ~/verify_hardware_env.sh && ~/verify_hardware_env.sh

## 9.2 Functional Smoke Test

Create and compile a minimal Zener design to confirm the end-to-end pipeline:

mkdir -p ~/designs/smoke_test && cd ~/designs/smoke_test

cat > smoke.zen << 'EOF'

# Minimal LED blink circuit smoke test

R1 = Component(symbol='Device:R', ref='R', value='330R',

footprint='Resistor_SMD:R_0402_1005Metric')

LED1 = Component(symbol='Device:LED', ref='D',

footprint='LED_SMD:LED_0402_1005Metric')

vcc = Net('VCC')

gnd = Net('GND')

R1[1] += vcc

R1[2] += LED1['A']

LED1['K'] += gnd

EOF

pcb build smoke.zen

echo 'Smoke test passed'

# 10. Quick Reference

## 10.1 Tool Stack by Use Case

| Use Case | Primary Tool | Supporting Tools |
| --- | --- | --- |
| Chip reference design from datasheet | Diode Zener + pcb CLI | pcbparts-mcp, kicad-happy/datasheets |
| Full board from text prompt | EasyEDA Pro MCP | pcbparts-mcp (LCSC search) |
| MCU board with device tree | Seeed KiCad MCP | kicad-happy/kicad |
| Python-defined circuit | circuit-synth | kicad-happy/kicad |
| DRC / BOM / Gerber export | mixelpixx KiCAD-MCP | kicad-happy/bom |
| Component sourcing only | pcbparts-mcp | @jlcpcb/mcp |
| FPGA / Verilog synthesis | EDA Tools MCP | iverilog, yosys |
| EMC pre-compliance review | kicad-happy/emc | kicad-happy/spice |

## 10.2 Repository Index

| Repository | URL |
| --- | --- |
| Diode pcb + Zener CLI | github.com/diodeinc/pcb |
| Diode reference designs | zener.diode.computer |
| Anthropic AI-EDA (jlc-mcp) | github.com/anthropics/ai-eda |
| mixelpixx KiCAD-MCP-Server | github.com/mixelpixx/KiCAD-MCP-Server |
| Seeed Studio KiCad MCP | github.com/Seeed-Studio/kicad-mcp-server |
| kicad-happy (Claude Code skills) | github.com/aklofas/kicad-happy |
| teileelektronik easyeda-mcp | github.com/teileelektronik/easyeda-mcp |
| pcbparts-mcp | github.com/Averyy/pcbparts-mcp |
| circuit-synth | github.com/circuit-synth/circuit-synth |
| claude-eda CLI | mcpxel.com/skills/claude-eda |

## 10.3 Known Limitations

- Diode pcb CLI Windows support is experimental — use WSL2 for all pcb commands

- mixelpixx KiCAD-MCP-Server requires KiCad to be installed on the same machine (Windows) with Python API accessible from WSL via PYTHONPATH

- EasyEDA Pro MCP bridge requires EasyEDA Pro desktop app running on Windows before MCP server is started — the WebSocket bridge will fail to connect otherwise

- circuit-synth auto-numbering: Python source is updated when KiCad auto-numbers refs — re-import after manual KiCad edits

- kicad-happy DigiKey and Mouser skills require paid API keys; LCSC, JLCPCB, Element14 are free

- AI-generated schematics from any tool in this stack require qualified engineering review before hardware fabrication — this applies to all safety-critical, medical, automotive, and aerospace applications

Document compiled May 2026 based on the Anthropic-Diode partnership blog post (claude.com/blog/making-claude-a-better-electrical-engineer), GitHub repositories, and MCP registry surveys. Verify all repo URLs and install procedures against upstream sources before running — the hardware EDA MCP ecosystem is evolving rapidly.